HHR OIL CHANGE

Use 5 quarts Mobil Super (High Mileage or 5000) 5W-30 oil

The torque specs for the oil cap and drain plug are both 25 Nm or 18 ft/lb.

Resetting Oil Life Monitor

- Turn the key into the on position (with the engine OFF).

- Press the INFO and RESET (arrow) buttons on the steering wheel at the same time, hold for a few seconds and let go. This will bring up the personalization menu.

- Once in the personalization menu, press the INFO button until you see OIL LIFE RESET.

- Press and hold the RESET button (arrow button) until you see ACKNOWLEDGED displayed. Your oil life monitor will read 100%.

- Turn the key to LOCK/OFF.